RISK ASSESSMENT / HAZID

[EQUIPMENT / SYSTEM / ACTIVITY NAME]

[Subtitle — e.g. Acceptance for Return to Service / New Equipment Introduction / Task Risk Assessment]

[Prepared in accordance with the Seadrill Risk Assessment Directive DIR-37-0147. Insert the specific industry/regulatory standards (e.g. API specifications) and equipment/quality documentation applicable to this assessment, and how conformance against them is verified.]

| Document Number | [FRM/Doc No.] |
| --- | --- |
| Revision | [Rev 00] |
| Date | [Month Year] |
| Classification | [Risk Level — Red / Yellow / Green — per Section 2.1; Confidential] |
| Equipment / Scope | [Equipment, system, or activity covered, including identification/serial numbers, plus RAMP criticality classification (SCE / ECE / OCE) if applicable] |
| Reference Standard(s) | DIR-37-0147 (Risk Assessment); DIR-00-0100 (RAMP); [applicable API / industry standard(s)] |
| Client / Project | [Client name – Contract / Rig] |
| Prepared by | [Author / Department] |

# Table of Contents

Table of Contents	1

Revision History	2

1. Introduction	2

1.1 Purpose	2

1.2 Scope and Applicability	2

1.3 Responsibilities	3

2. Requirements	3

2.1 Risk Matrix	3

2.1.1 Risk Evaluation Step 1 - Consequence (P.E.A.R)	3

2.1.2 Risk Evaluation Step 2 - Frequency	3

2.1.3 Acceptance Criteria	4

2.2 Hierarchy of Controls	4

2.3 Task Process (PIMED) and Risk Assessment Levels	4

3. Equipment and Standards Basis	5

3.1 [Applicable Standard(s)]	5

3.2 Equipment Specification and Quality Evidence	5

3.3 Alignment with the Rig Asset Management Platform (RAMP, DIR-00-0100)	5

4. HAZID Risk Register	8

5. Summary of Risk Reduction Measures	12

6. ALARP Demonstration	13

7. Conclusion and Recommendation	13

8. Document Control and References	13

Approval Signature Block	14

## Revision History

| Rev | Date | Description of Change | Author |
| --- | --- | --- | --- |
| 01 | [Month Year] | Initial Issue | [Author] |

# 1. Introduction

## 1.1 Purpose

[The purpose of this Risk Assessment / HAZID is to identify the hazards and manage the potential risks associated with <describe the equipment, system, or activity and what is being done to/with it — e.g. acceptance for return to service, introduction of new equipment, a task or process change>. It is prepared in accordance with the Seadrill Risk Assessment Directive (DIR-37-0147), evaluates risk using the Seadrill Risk Matrix (FRM-37-0138), and — where the subject is equipment — aligns its management with the Rig Asset Management Platform (RAMP, DIR-00-0100). State the equipment/activity’s criticality (e.g. Safety and/or Environmentally Critical Element (SECE)) and the Major Accident Hazard(s) it could contribute to if it fails.]

[State the specific objective — e.g. to identify the applicable requirements of the relevant industry/regulatory standard(s) and to demonstrate, on a clause-by-clause basis, how the supplied equipment/quality documentation verifies those requirements have been met, while flagging any aspects requiring further verification.]

## 1.2 Scope and Applicability

[State exactly what this risk assessment applies to, including any identification/serial numbers.] Where Client requirements or local regulations differ from this assessment or the governing Directive, the most stringent requirements shall be followed. It covers:

[Structural / mechanical integrity of primary load-bearing or safety-critical components;]

[Functional / performance testing relevant to the equipment or activity (e.g. proof load, pressure, hydrostatic, positive-lock or barrier function tests);]

[Material traceability, mechanical properties, replacement parts / bolting, and any repair or reclamation processes used;]

[Certification, quality assurance, and documentation traceability, including any facility accreditation (e.g. API Q1 / ISO 9001) and cost/quality-plan considerations relevant to acceptance;]

[Registration of the equipment/asset within the Rig Asset Management Platform (RAMP, DIR-00-0100) — criticality classification (SCE / ECE / OCE), Performance Standard, certificate tracking and re-inspection interval, where applicable.]

[State what is explicitly out of scope.] It does NOT cover the wider system or the field/offshore execution of the activity, which shall be controlled at the worksite through a Task Based Risk Assessment (TBRA, FRM-37-0015) and/or 5-Point Check per DIR-37-0147.

## 1.3 Responsibilities

[Name the Technical Authority / discipline owner] is responsible for the content of this assessment and for the acceptance decision. [Name the Rig Manager / OIM / Section Leader role] is responsible for ensuring verification actions are completed and recorded. [Name any third-party / service provider / quality representative] is responsible for providing supporting evidence. All actions shall be transferred to Synergi with defined owners and close-out dates.

# 2. Requirements

Hazards associated with the subject of this assessment pose a level of risk that shall be evaluated and managed to ALARP using the Seadrill Risk Matrix (FRM-37-0138).

## 2.1 Risk Matrix

## 2.1.1 Risk Evaluation Step 1 - Consequence (P.E.A.R)

Potential consequences are evaluated using the P.E.A.R hierarchy (People, Environment, Asset, Reputation) and ranked from 1 (high) to 5 (low) as required by the Risk Matrix. [State the credible worst-case consequence of the hazard being realised, and the resulting Consequence ranking.]

## 2.1.2 Risk Evaluation Step 2 - Frequency

The likelihood of a hazard being realised is evaluated using the following classifications:

A - Frequent (multiple annual events on the rig)

B - Likely (annual event on the rig)

C - Unlikely (annual event in the company)

D - Rare (annual event in the industry)

E - Very Rare (decade event in the industry)

## 2.1.3 Acceptance Criteria

| Red Risk | Unacceptable. The tool shall not be accepted for service while the untreated risk lies in this area; controls must be implemented and the risk reduced to at least Yellow. |
| --- | --- |
| Yellow Risk | Additional controls are required; acceptance may proceed once the risk has been reduced to ALARP, even if it remains Yellow. |
| Green Risk | Generally acceptable; additional controls considered only where reasonably practicable. |

The risk for all aspects shall be reduced to As Low As Reasonably Practicable (ALARP) before [the equipment is returned to service / the activity proceeds]. The matrix below applies (Consequence 1 = high; Frequency A = most frequent):

| Consequence / Frequency | E - Very Rare | D - Rare | C - Unlikely | B - Likely | A - Frequent |
| --- | --- | --- | --- | --- | --- |
| 1 - Fatality / Major (high) |  |  |  |  |  |
| 2 - Lost Time Incident (LTI) |  |  |  |  |  |
| 3 - Medical Treatment (MTC) |  |  |  |  |  |
| 4 - Minor / First Aid |  |  |  |  |  |
| 5 - Negligible (low) |  |  |  |  |  |

## 2.2 Hierarchy of Controls

Controls (risk mitigation) are applied following the Hierarchy of Controls, evaluated from the top (most effective) down:

Elimination - remove the hazard (e.g. reject and replace the tool rather than repair where defects exceed allowable limits);

Substitution - replace with a lower-risk alternative (e.g. OEM-supplied certified replacement components and bolting);

Engineering - design-based barriers (e.g. proof-load testing to 1.5x rated load, hard-chrome reclamation, NDE, positive lock indication);

Administrative - procedures, competency and documentation (e.g. WPS/PQR, Data Book / Certificate of Conformance, TA acceptance, MoC);

PPE - last line of defence during handling and testing operations.

In this assessment the principal controls are Engineering (NDT, dimensional, proof-load and hydrostatic testing, hard-chrome reclamation) and Administrative (qualified procedures, certification and TA acceptance). The proposed risk reducing measures in Section 4 are framed accordingly.

## 2.3 Task Process (PIMED) and Risk Assessment Levels

This document is an engineering / equipment acceptance HAZID (or task-level risk assessment) supporting the decision. Any subsequent field/offshore activity shall follow the Seadrill Task Process (PIMED — Plan, Identify, Manage, Execute, Debrief) and be controlled by a Task Based Risk Assessment (TBRA, FRM-37-0015) and/or 5-Point Check, as required by DIR-37-0147. Lessons learned from acceptance and first use shall be captured at Debrief and recorded in Synergi where substantive.

# 3. Equipment and Standards Basis

## 3.1 [Applicable Standard(s) — e.g. API Specification/Recommended Practice]

[Summarise the controlling clauses of each applicable industry/regulatory standard supplied in this project’s 02_API Guidelines folder. State, clause by clause, what each requires (loading/design factors, testing, materials, marking, documentation) — this becomes the “API basis” referenced against “Existing Risk Controls” in the HAZID Risk Register.]

## 3.2 Equipment Specification and Quality Evidence

[Summarise the equipment/OEM specification supplied in 01_Equipment Specification, and the inspection, test, certification and quality-control evidence supplied in 03_Quality Plan & Costs. Identify the manufacturer/service provider, any facility accreditation (e.g. API Q1, ISO 9001), and how each piece of evidence maps to a standard’s requirement identified in Section 3.1. Note any cost implications of proposed risk reducing measures where relevant to the acceptance decision.]

## 3.3 Alignment with the Rig Asset Management Platform (RAMP, DIR-00-0100)

[Where the subject of this assessment is equipment, describe how it is/will be managed within RAMP:]

Equipment identification — asset/location record in Maximo (SAMS) under the Standard Equipment Structure, tracked by serial number;

Criticality and Performance Standard — classification as Safety Critical Equipment (SCE) / Environmental Critical Equipment (ECE) / Operational Critical Equipment (OCE), with SECE Group/Tag and Performance Standard populated in Maximo where applicable (see Knowledge Digest §5);

Third-party / service provider — managed under DIR-00-0239, with the work scope under Management of Change (DIR-37-0015) where the equipment/task changes an existing maintenance or design basis;

Certification tracking — certificate and expiry dates recorded in Maximo, with supporting documents filed in eDocs (DIR-37-0019);

Maintenance cycle — the ongoing PM/inspection interval this assessment establishes or confirms.

# 4. HAZID Risk Register

Requirements from the applicable standard(s) (Section 3.1) are identified under "Existing Risk Controls" together with the supplied equipment/quality evidence (Section 3.2); "Proposed Risk Reducing Measures" capture the additional verification or action needed, following the Hierarchy of Controls (Section 2.2). Consequence (Con) 1 = high; Frequency A = most frequent, E = very rare (per DIR-37-0147). The two rows below (EX-1, EX-2) are worked examples retained from a real Seadrill HAZID for illustration — replace them with the actual hazards for this assessment and renumber sequentially.

| ID | HAZID Guideword / Reference | Source of Hazard | Threat / Cause of Loss of Control | Existing Risk Controls | Con | Freq | Risk | Proposed Risk Reducing Measures | Action (transfer to Synergi) | Con | Freq | Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EX-1 | Structural Failure of Primary Load Path API 16F S.14.3.3 / 16Q S.4.13.3 | Re-certified Shaffer FT-H 1000T running tool primary load-bearing members (housing PN 20035034, stem, end shoe PN 20035035, lock dogs) carry the full riser string and BOP stack. Pre-existing service-induced corrosion, fatigue or wear may not be fully detected/removed during overhaul. | Undetected crack or material loss reduces capacity below the SF_D = 2.25 design margin of API 16F S.14.3.3.3; structural failure during deployment -> dropped riser string and BOP stack, dropped object on personnel, loss of well containment. | API 16F S.14.3.3 requires analysis of yielding/buckling/deflection/rupture with design safety factor 2.25. VRV: 100% visual + high-contrast MT and PT over housing, cylinder, piston and stem; Damage Report RG-OP-032 No.269/25 records per-component inspection, laudo and rejection criteria; engineering defect assessment defines repair scope. | 1 | D | Yellow | Confirm NDT acceptance criteria documented to ASTM E709 (MT) / ASTM E165 (PT) per API 16F S.6; re-inspect all load-path welds post-repair; retain MT/PT reports in Data Book; Seadrill TA to review engineering defect assessment against OEM allowable repair limits before acceptance. | Maintenance / TA to verify NDT reports and acceptance criteria in VRV Data Book prior to return-to-service tag. | 1 | E | Green |
| EX-2 | Positive Lock / Engagement Indication (GAP) API 16F S.14.3.2 & S.14.4.1.2 | API 16F requires the riser running tool to provide a positive lock signal output and a visual indication confirming the lock/latch is engaged prior to lifting. The VRV overhaul scope (inspect / repair / test) does not explicitly evidence the S.14.4.1.2 positive-lock-indication functional test. | Lifting on a tool that is not fully latched (false-positive, or absent lock indication) -> riser/BOP release -> dropped object / loss of containment. | Lock dogs (6 ea) polished/inspected and function confirmed via hydraulic function and hydrostatic testing (Quotation item 8.7). API 16F S.14.3.2 / S.14.4.1.2 requirement noted; no explicit functional lock-indication test evidenced in current package. | 1 | C | Red | Confirm whether the FT-H design provides a positive lock-indication method; if fitted, perform the S.14.4.1.2 functional verification (true-positive when engaged; no false-positive when not engaged) and document it; if the tool is a flanged-bolt type (exempt per S.14.3.2), confirm engagement is verified by a controlled offshore procedure. Seadrill TA to close out before return to service. | TA WCE to confirm lock-indication provision/exemption and document functional verification or alternative control. | 1 | E | Green |

# 5. Summary of Risk Reduction Measures

| ID | Hazard | Key Risk Reducing Measure | E.Con | E.Freq | E.Risk | Action Owner | M.Con | M.Freq | M.Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EX-1 | Structural integrity of primary load path | Document MT/PT acceptance criteria & re-inspect load-path welds; TA review of defect assessment vs OEM limits | 1 | D | Yellow | TA / Maintenance | 1 | E | Green |
| EX-2 | Positive lock indication (GAP) | Confirm lock-indication provision/exemption; perform S.14.4.1.2 functional verification or define offshore control | 1 | C | Red | TA WCE | 1 | E | Green |

# 6. ALARP Demonstration

[State the overall ALARP conclusion, e.g.: All residual risks following implementation of the proposed risk reducing measures have been assessed as Green (Acceptable) or low-Yellow (ALARP). The demonstration rests on the following:]

[Standard compliance: identify the specific design factors, test levels, or acceptance criteria from the applicable standard(s) that are verified as met;]

[Barrier integrity: state how the SECE/critical function is verified — e.g. through NDT, dimensional inspection, and functional/pressure testing;]

Hierarchy of Controls: engineering controls (testing, NDE) and administrative controls (qualified procedures, certification, Technical Authority acceptance) are applied in preference to reliance on PPE;

[Independent assurance: identify any accredited facility/quality system (e.g. API Q1, ISO 9001) or third-party report supporting the evidence;]

[Gap closure: list any open items (referencing HAZID Register entry numbers) that must be closed before acceptance;]

Governance: acceptance is governed through Seadrill engineering acceptance / MoC (DIR-37-0014) with Technical Authority sign-off, and all actions transferred to Synergi.

# 7. Conclusion and Recommendation

[State the overall conclusion — e.g.: This HAZID concludes that the supplied evidence addresses the principal requirements of the applicable standard(s), provided the verification evidence is obtained/reviewed and the identified gaps are closed. The highest-priority actions before acceptance/execution are:]

Priority 1 (Red — Entry [X]): [Describe the highest-priority action, referencing the HAZID Register entry number, and who must sign it off.]

Priority 2 (Entries [X, Y]): [Describe the next-priority actions.]

Priority 3 (Entries [X, Y, Z]): [Describe any remaining actions.]

All risk reducing measures shall be transferred to Synergi as formal action items with defined owners and close-out dates, and verified complete before [the equipment is returned to service / the activity proceeds]. Any subsequent field activity shall be controlled by TBRA / 5-Point Check per DIR-37-0147.

# 8. Document Control and References

| Document / Reference | Title / Description |
| --- | --- |
| DIR-00-0100 (v8.06) | Seadrill Rig Asset Management Platform (RAMP) - technical integrity & asset management (equipment ID, criticality, rotating assets, riser portal, work management) |
| DIR-37-0147 (v2.02) | Seadrill Risk Assessment Directive (governing process, risk matrix, hierarchy of controls, PIMED) |
| DIR-00-0034 | Major Accident Hazard Management |
| DIR-37-0015 | Management of Change Requests |
| DIR-00-0239 | Management of Service Providers Onboard |
| DIR-37-0019 | Management of Technical Documentation (eDocs) |
| DIR-37-0011 | Barrier Management |
| FRM-37-0138 | Seadrill Risk Matrix |
| FRM-37-0015 | Task Based Risk Assessment (TBRA) |
| FRM-00-0034 | PIMED — Task Process Workflow |
| SYS-00-0033 / SYS-00-0034 | Reliability Centred Maintenance (RCM) process / RAMP Equipment Structure and Coding |
| [Applicable API / industry standard reference] | [Title / edition / section — add one row per standard used in Section 3.1] |
| [Equipment / quality documentation reference] | [OEM manual, inspection report, certificate, or quotation — add one row per document used in Section 3.2] |

## Approval Signature Block

This risk assessment does NOT require printing and attaching to a hard copy of the TBRA. Digital signature via DocuSign is the approved method of sign-off.

| Name | Title | Signature | Date |
| --- | --- | --- | --- |
| [Name] | Technical Authority |  |  |
| [Name] | [Title] |  |  |
| [Name] | [Title] |  |  |